Agri-Pulse Decision Engine: Technical Specification

Document ID: SPEC-AGRI-AI-002 | Execution Environment: Local Model Inference (Model TBD)

1. Architectural Philosophy & Separation of Concerns

The AI decision engine functions as a tactical operational arbitrator, not a mathematical calculator. Large Language Models frequently make calculation errors when performing multi-step arithmetic, floating-point balance accounting, or aggregating 168-hour weather vectors.

To guarantee zero mathematical hallucination and achieve rapid, reliable local execution:
1. Python executes all physics, hydrology, and meteorology: It ingests weather data, computes reference evapotranspiration (ET0) via FAO-56 Hargreaves equations, models root-zone soil water balance, and pre-calculates net hydrological deficit in millimeters.
2. The Local Model evaluates agronomic context and timing: It receives a normalized, pre-computed JSON summary and selects the tactical action enum, the non-peak execution window, and the crop-protection biological rationale.
3. Local Model Deployment: Instead of querying external cloud APIs, inference runs locally on the host machine using an abstracted local inference server (e.g., Ollama, vLLM, or llama.cpp OpenAI-compatible endpoint). This eliminates cloud API keys, network transit overhead, cloud rate limits, and external downtime.

┌────────────────────────────────────────────────────────┐ │ RAW INGESTION (UI Inputs + 168h Open-Meteo Weather) │ └──────────────────────────┬─────────────────────────────┘ │ ▼ ┌────────────────────────────────────────────────────────┐ │ DETERMINISTIC PRE-PROCESSING (Python / NumPy) │ │ • FAO-56 Hargreaves ET₀ & ET_c calculation │ │ • Soil hydraulic lookup (Field Capacity, Wilting Pt) │ │ • 7-day unmitigated water depletion projection │ │ • Net hydrological deficit calculation (mm) │ └──────────────────────────┬─────────────────────────────┘ │ ▼ ┌────────────────────────────────────────────────────────┐ │ LOCAL MODEL DECISION ENGINE (Local Runner - Model TBD)│ │ • Provider: Local HTTP runner (Ollama / vLLM / local) │ │ • Input: Compact, pre-calculated JSON state │ │ • Constraints: Enum candidate actions & zero math │ │ • Anchors: 3 few-shot operational calibration cases │ │ • Output: Strictly typed JSON conforming to schema │ └──────────────────────────┬─────────────────────────────┘ │ ▼ ┌────────────────────────────────────────────────────────┐ │ POST-INFERENCE GUARDRAILS (Python Validation) │ │ • Physical bound checks (0 mm on HOLD, night window) │ │ • Instant fallback to rule-matrix if timeout / error │ └──────────────────────────┬─────────────────────────────┘ │ ▼ ┌────────────────────────────────────────────────────────┐ │ DISPATCH EMISSION (Sent to React Frontend Console) │ └────────────────────────────────────────────────────────┘

2. Deterministic Pre-Processing (Python Engine)

Before invoking the local model, Python executes four deterministic steps in <2 ms:

2.1 Soil & Phenology Normalization

Soil Hydraulic Constants: Sandy Loam (θFC = 20.0%, θPWP = 9.0%), Loam (θFC = 27.0%, θPWP = 12.0%), Clay (θFC = 35.0%, θPWP = 20.0%).

Crop Growth Stages: Sets effective root depth Zr (Wheat at Flowering = 0.60 m), crop coefficient Kc (Wheat at Anthesis = 1.15), and physiological vulnerability (e.g., EXTREME_POLLEN_STERILITY).

2.2 Evapotranspiration & Hazard Detection

• Reference ET0: Calculated via temperature-difference Hargreaves method:
  ET0(t) = 0.0023 · Ra · (Tmean + 17.8) · sqrt(Tmax - Tmin)
• Crop Water Demand: ETc(t) = Kc · ET0(t)
• Heatwave Flag: Tmax >= 38.0°C for 2 or more consecutive days.

2.3 Simulated Depletion & Precomputed Deficit

• Daily Moisture Balance: θ(t) = θ(t-1) - [(ETc(t) - P(t)) / (Zr · 1000)] × 100
• Wilting Point Crossing: Flags projected_failure_day when θ(t) <= θPWP.
• Precomputed Deficit (mm): Deficit = max(0.0, (0.90 · θFC - θcurrent) · Zr · 10). Restores moisture to 90% of Field Capacity without saturation.

3. Local Model Input Contract

Compact JSON payload serialized by Python and provided into local model context:

{ "crop_context": { "crop": "Wheat", "growth_stage": "Flowering / Anthesis", "thermal_threshold_c": 38.0, "primary_heat_vulnerability": "Pollen sterility and flower abortion" }, "hydrology_summary": { "soil_type": "Sandy Loam", "current_moisture_pct": 14.0, "field_capacity_pct": 20.0, "wilting_point_pct": 9.0, "precomputed_deficit_mm": 24.0, "simulated_depletion": { "crosses_wilting_point": true, "projected_failure_day": 2, "hours_until_stress_onset": 38 } }, "hazard_profile": { "is_heatwave_active": true, "peak_temperature_c": 41.5, "baseline_seasonal_avg_c": 33.0, "heatwave_duration_days": 3, "days_until_peak": 2, "total_forecasted_rain_mm": 0.0 }, "irrigation_system": "Furrow_Flood", "candidate_decisions": [ "HOLD_CURRENT_LEVEL", "PREEMPTIVE_NIGHT_PRECHARGE", "SPLIT_CYCLE_LIGHT_IRRIGATION", "EMERGENCY_CANOPY_COOLING" ] }

4. Local Model System Prompt & Calibration Anchors

You are Agri-Pulse, an expert autonomous agronomic dispatch officer and thermal plant physiologist. ### OBJECTIVE Evaluate pre-digested soil hydrology, phenological vulnerabilities, and heatwave hazards to select an optimal irrigation directive. ### HARD OPERATIONAL CONSTRAINTS 1. DO NOT CALCULATE ARITHMETIC: Math is precomputed. If HOLD_CURRENT_LEVEL, water_depth_mm MUST be 0.0. For active watering, water_depth_mm MUST equal precomputed_deficit_mm (or fractional split for pulse cycles). 2. TIMING ENFORCEMENT: Never irrigate during peak daytime heat (10:00 - 18:00). PREEMPTIVE_NIGHT_PRECHARGE must execute between 22:00 and 05:00. 3. PHENOLOGICAL SENSITIVITY: Flowering/Anthesis is highly vulnerable to pollen sterility at >38°C; Vegetative can tolerate mild deficit. 4. STRICT JSON OUTPUT: Return ONLY valid JSON matching schema. ### CALIBRATION ANCHORS • Wheat Flowering + 41.5°C -> CRITICAL_HEAT_DEFICIT / PREEMPTIVE_NIGHT_PRECHARGE (24mm, 22:00 - 04:30 PKT) • Cotton Vegetative + 39.5°C (Clay) -> SUFFICIENT_NO_ACTION / HOLD_CURRENT_LEVEL (0mm, avoids hypoxia) • Tomato Fruit Set + 40.0°C (Loam) -> CRITICAL_HEAT_DEFICIT / SPLIT_CYCLE_LIGHT_IRRIGATION (14mm, pulsed cycle)

5. Output Data Contracts (Pydantic Models)

class SufficiencyStatus(str, Enum): SUFFICIENT_NO_ACTION = "SUFFICIENT_NO_ACTION" MODERATE_DEFICIT = "MODERATE_DEFICIT" CRITICAL_HEAT_DEFICIT = "CRITICAL_HEAT_DEFICIT" class CandidateDecision(str, Enum): HOLD_CURRENT_LEVEL = "HOLD_CURRENT_LEVEL" PREEMPTIVE_NIGHT_PRECHARGE = "PREEMPTIVE_NIGHT_PRECHARGE" SPLIT_CYCLE_LIGHT_IRRIGATION = "SPLIT_CYCLE_LIGHT_IRRIGATION" EMERGENCY_CANOPY_COOLING = "EMERGENCY_CANOPY_COOLING" class ActionPlan(BaseModel): water_depth_mm: float execution_window: str biological_rationale: str = Field(..., max_length=220) class DecisionOutput(BaseModel): sufficiency_status: SufficiencyStatus selected_decision: CandidateDecision decision_confidence: float action_plan: ActionPlan

6. Python Service Implementation (Local Model Client)

async def query_local_model(state: Dict[str, Any]) -> DecisionOutput: url = f"{settings.LOCAL_MODEL_BASE_URL.rstrip('/')}/v1/chat/completions" payload = { "model": settings.LOCAL_MODEL_NAME, "messages": [ {"role": "system", "content": SYSTEM_INSTRUCTION}, {"role": "user", "content": json.dumps(state)} ], "temperature": 0.05, "response_format": {"type": "json_object"} } async with httpx.AsyncClient(timeout=settings.LOCAL_MODEL_TIMEOUT_SECONDS) as client: response = await client.post(url, json=payload) response.raise_for_status() raw_json = response.json() content = raw_json["choices"][0]["message"]["content"] return DecisionOutput.model_validate_json(content) def post_inference_guardrails(decision: DecisionOutput, state: Dict[str, Any]) -> DecisionOutput: if decision.selected_decision == CandidateDecision.HOLD_CURRENT_LEVEL: decision.action_plan.water_depth_mm = 0.0 if decision.selected_decision == CandidateDecision.PREEMPTIVE_NIGHT_PRECHARGE: if any(h in decision.action_plan.execution_window for h in ["11:", "12:", "13:", "14:", "15:", "16:", "17:"]): decision.action_plan.execution_window = "Tonight 22:30 - 04:30 PKT (Night Only)" if decision.action_plan.water_depth_mm > 45.0: decision.action_plan.water_depth_mm = 45.0 return decision

7. Local Model Configuration & Runtime Options

The local decision engine is decoupled from any specific model vendor. The model will be chosen later based on hardware:
• Inference Server Options: Ollama (http://localhost:11434) with JSON format mode; vLLM or llama.cpp server (http://localhost:8000/v1) with grammar/schema constrained decoding.
• Candidate Local Model Profiles: Compact edge models (Llama 3.2 3B-Instruct, Qwen 2.5 3B-Instruct, Phi-3.5 Mini 3.8B) or mid-tier models (Mistral 7B-Instruct v0.3, Llama 3.1 8B-Instruct, Qwen 2.5 7B-Instruct).
• Configuration: LOCAL_MODEL_BASE_URL (default: http://localhost:11434), LOCAL_MODEL_NAME (model-tbd), LOCAL_MODEL_TIMEOUT_SECONDS (0.50s max timeout before instant fallback).

8. Verification Matrix

| Case ID | Input State Scenario | Expected Decision | Guardrail Verification |
| --- | --- | --- | --- |
| VM-01 | Wheat, Flowering, Sandy Loam, Moisture 14%, Heatwave 41.5°C in 48h | PREEMPTIVE_NIGHT_PRECHARGE | water_depth_mm equals deficit (24.0 mm); window restricted to 22:00 - 05:00. |
| VM-02 | Cotton, Vegetative, Clay, Moisture 32%, Heatwave 39.5°C in 24h | HOLD_CURRENT_LEVEL | water_depth_mm forced to 0.0; flags soil aeration and hypoxia protection. |
| VM-03 | Tomato, Fruit Set, Loam, Moisture 13%, Heatwave 40°C in 12h | SPLIT_CYCLE_LIGHT_IRRIGATION | Fractional split volume assigned; avoids peak daytime evaporative loss. |
| VM-04 | Local runner offline / timeout (>500 ms) / bad JSON | DETERMINISTIC_FALLBACK | Recovers in <1 ms; tagged engine_source: DETERMINISTIC_FALLBACK. |